Moment of a force: Part 1

The moment of a force about a point is defined as the force magnitude times the perpendicular distance from the point to the line of action of the force. The moment is a measure of the tendency of the force to cause rotation.

Consider the force F acting on the handle of the wrench.

The magnitude of the moment of a force:

MO: Magnitude of the moment of F around point O

d: Perpendicular distance from O to the line of action of F

Direction of the moment in 2-D: The direction of the moment is given by the right hand rule: Counter Clockwise (CCW) is out of the page, Clockwise (CW) is into the page.

Example: By closing the lid of a bottle, you are imposing a clockwise moment.

Calculating the moment in 2-D using components:

- Select a positive direction (CCW or CW),

- Calculate each moment and add them, using the proper sign for each term,

- Always remember to write the unit of moment which is Nm.

Example:

In the following figure, calculate the moment about the point O:

- We choose the CCW as positive direction for moment,

- Moment of component of F along x about O is Fx times the perpendicular distance from O (or d1), which is clockwise, so it is

- Moment of component of F along y about O is Fy times the perpendicular distance from O (or d2), which is counter clockwise, so it is

- Moments add together as vectors, so the total moment is:

Example:

In the following figure, if  is 60 degrees and r is 30 mm and F is 6 N, what is the magnitude of the moment about O.

- We choose the CCW as positive direction for moment,

- Component of F along r (or F||) produces no moment, since it passes from point O.

If a force passes through a point, it produces no moment about that point!

Component of F perpendicular to r (or )

produces the moment .

So the total moment of F about O is:

.

Note: moving a force along its line of action does not change its moment

Remember:

The moment about O is also calculated using the magnitude of force F times perpendicular distance from O to the line of action of F which is d:

.