Forces and Moments: Part 3

Moment about an axis:

Sometimes the moment about a point is known and you are supposed to calculate its component about an axis. To find the moment, consider the dot product of Mo and unit vector along axis a:

O: any point on a-a

You can also find the tangent force Fθ and then r x Fθ is the moment about aa:

Let us consider the following example:

| Force F causes a moment MO about point O. What is the component of MO along axis oy (My)? Solution: |  |
| --- | --- |

There are 2 methods to find My

- Scalar Analysis

- Vector Analysis

(1) Scalar Analysis (first way):

MO=(20)(0.5)=10 N.m

MO tends to turn the pipe around axis ob. The component of MO along the y-axis, My, tends to unscrew the pipe from the flange at O.

Thus it is important to know its value.

My = (3/5)(10) = 6 N.m

Scalar Analysis (second way):

To find My directly (not form MO) it is necessary to determine the moment-arm, knowing that the distance from F to the y-axis is 0.3m:

My = (20)(0.3) = 6 N.m

In general, If the line of action of a force F is perpendicular to any specific axis aa thus:

Ma = F .da

(2) Vector Analysis:

First, use the cross product formula to calculate the moment about O:

MO = rA × F

MO = (0.3i + 0.4j) ×  (-20k)

MO = {-8i + 6j} N.m

Then use the dot product of MO and the unit vector along y-axis to get My:

My = MO . ua

My = (-8i + 6j). (j)

My = 6 N.m