Forces and Moments: Part 7

Reduction to a wrench:

In the general case, the resultant moment (MRo) is perpendicular to FR.

Now consider the case where FR acts at an angle θ to MRo (Fig. a). Resolve MRo into two components (Fig. a):

M|| || FR and M┴ ┴FR,

Eliminate M┴ by moving FR by distance d = M┴/ FR from point O to point P. Now we are left with FR at P and M|| at O (Fig. b). Since M|| is a free vector which can be moved to P (Fig. c)

This combination of a collinear force and a couple moment is called a wrench or screw.

The axis of the wrench has the same line of action as the force. The wrench tends to cause both translation along and rotation about this axis.

| (a) | (b) | (c) |
| --- | --- | --- |