Problem 1:

| The beam is adjusted to the horizontal position by means of a wedge located at its right support. If the coefficient of static friction between the wedge and the two surfaces of contact is μs = 0.25, determine the horizontal force P required to push the wedge forward. Neglect the weight and size of the wedge and the thickness of the beam. Solution: If the wedge is on the verge of moving to the right, then slipping will have to occur at both contact surfaces. FB = μsNB = 0.25NB FC = μsNC = 0.25NC |  |
| --- | --- |

Problem 2:

| If the beam AD is loaded as shown, determine the horizontal force P which must be applied to the wedge in order to remove it from under the beam. The coefficients of static friction at the wedge’s top and bottom surfaces are μC-A = 0.25 and μC-B = 0.35, respectively. If P=0, is the wedge self-locking? Neglect the weight and size of the wedge and the thickness of the beam. Solution: The equivalent force for triangular distributed force is (1/2)(4)(3)=6 and also for the uniform one (4)(4)=16 If the wedge is on the verge of moving to the right, then slipping will have to occur at both contact surfaces. FA = μC-ANA = 0.25NA FB = μC-BNB = 0.35NB |  |
| --- | --- |

Beam:(+MD=0NAcos10°(7)+0.25NAsin10°(7)−(6)(2)−16(5)=0⇒NA=12.78kNWedge:+↑Fy=0NB−12.78(sin80°)−0.25(12.78)(sin10°)=0⇒NB=13.14kN+Fx=0P+(12.78)(cos80°)−0.25(12.78)(cos10°)−0.35(13.14)=0⇒P=5.53kN

Since a force P = 5.53 kN > 0 is required to pull out the wedge, the wedge will be self-locking when P = 0.